





















































	GB-60�PRIVATE ��





	GROUND-LOOP BLOCKER


	














	INSTRUCTION BOOK











	IB6329-01


	




































































	TABLE OF CONTENTS

















DESCRIPTION									2











MOUNTING INSTRUCTIONS							2











HOW TO CABLE THE GB-60							2











POWER SUPPLY INSTALLATION						     3











OPERATION										3











CARE AND MAINTENANCE							     3











APPLICATIONS (WHERE TO USE THE SYSTEM)				3




































































GB-60.ISB									PAGE 1 OF 3


DESCRIPTION





	This product blocks and removes the most common problems in CCTV video installations. The GB-60 also restores the picture quality before it goes to the monitor or recorder. The unit is placed at the monitor site or anywhere a video problem exists. Ground-Loop Bars in CCTV pictures caused by AC power ground-loops are cancelled completely. 





	The GB-60 will eliminate ground-loop problems induced by lightning protection equipment. It also corrects for video level loss and loss of picture definition caused by long cable runs and twisted pair transmission systems up to 3000 feet. The unit prevents Ghosting in the picture by absorbing and eliminating the reflected video ghost signal. It will also restore the correct impedance to down-stream video signals to prevent secondary ghosting in the picture. 





	Longer cable runs can be accommodated by placing additional GB-60 units at intermediate locations to be used as line extenders. In this way, a mile or more of cable loss can be corrected and 60 cycle bars found particularly on long cable can be eliminated.





	Each GB-60 corrects up to 4 CCTV channels with each channel having independent video level and definition controls as well as independent ground-loop blocking.  A second fully isolated video output is offered as an option for systems that require  additional monitor locations, so the GB-60 will operate as a Distribution Amplifier. 








MOUNTING INSTRUCTIONS





	The rugged one piece mounting structure allows you to mount the unit firmly in place with two screws. Select a place to mount the unit away from harsh or wet environments, indoors is recommended. The GB-60 should be located near the monitor equipment or the place you wish to bring the video signal to. Select a position that gives you the best access to cable the system and reduces the labor in installation.





HOW TO CABLE THE GB-60





	Attach the cable coming from the camera to the Video Input BNC. Then attach a cable from the BNC connector marked Video Output to the monitor equipment. BE SURE TO TERMINATE THE END OF THE VIDEO CABLE WITH A 75 OHM TERMINATION OR PROPERLY TERMINATE INTO OTHER EQUIPMENT. Repeat this step for each channel you wish to improve. 











GB-60.ISB								PAGE 2 OF 3








POWER SUPPLY INSTALLATION





	The GB-60 is powered by a 24 VDC wall mount power transformer. Plug the 24 VDC connector into the jack marked PWR. The Green LED will illuminate when the power is applied.








OPERATION





	When video is applied to the input connectors of the GB-60 an internal active video clamp blocks the 60 Hz ground-loop current from interfering with the CCTV video picture. Next the video goes into an adjustable gain stage that lets the installer adjust for cable loss on long cable runs. A second control allows the installer to adjust for high frequency losses that cause reduction in picture quality. The down stream video impedance is then matched to the 75 Ohm cable to prevent Ghosting downstream.








CARE AND MAINTENANCE





	There is no routine maintenance or calibration required with this equipment. There are no field adjustable controls inside the box. Opening the box will void your warranty. All adjustments are made through the front panel.








APPLICATIONS (WHERE TO USE THE SYSTEM)





	This system can be used anywhere that a video signal in coax cable exists. It can be used to improve Twisted pair transmission system response. Video pictures on long cable runs will benefit from the installation of the GB-60. In a long cable run the GB-60 can be placed at the mid point between the camera and the monitor to improve the transmission level of the signal.
























































GB-60.ISB									PAGE 3 OF 3


�











 





 











