TVP-20

CCTV VIDEO-PLEXER

INSTRUCTION BOOK

IB629301

11-1-95

TABLE OF CONTENTS

DESCRIPTION									3

MOUNTING INSTRUCTIONS							3

HOW TO CABLE THE TVP-20							3

HOW TO CABLE THE RVP-20							4

POWER SUPPLY INSTALLATION						4

OPERATION										4

CARE AND MAINTENANCE							4

APPLICATIONS (WHERE TO USE THE SYSTEM)				4

APPLICATIONS (LONG DISTANCE VIDEO ON COAX)			4

TVP-20.ISB								PAGE 2 OF 4

DESCRIPTION

These devices are used to transmit two CCTV Camera signals over a single Coaxial Cable at the same time. This system can be used to Add-On a second camera in an existing Coaxial camera system without the need to run additional cable. It can also be used to improve transmission of CCTV pictures over long cable runs. The system is simple to install and convenient. It will save you the expense of running a new cable back to the monitor point just to add one more camera to your system. Any video cable in your system can carry two video signals back to the monitor location.

The TVP-20 TRANSMITTER VIDEO-PLEXER is the transmitter module used to Encode and transmit the two video signals. Simply connect the two cameras to the input connectors, connect the power supply and use a single cable back to the monitor site.

The RVP-20 RECEIVER VIDEO-PLEXER is the receiver module used to receive and  decode the two video signals. Simply connect the cable coming from the TVP-20 to the cable input connector, then take camera A and B outputs to your Quad, Switcher, Monitors, etc. Connect the power supply and you are ready to go.

The system is housed in a black ABS enclosure that has a UL flame rating of 94-VO and is powered by any 24VAC power transformers. The system has an LED power indicator light to help installation.

MOUNTING INSTRUCTIONS

The rugged one piece mounting structure allows you to mount the unit firmly in place with two screws. Select a place to mount the unit away from harsh or wet environments, indoors is recommended. The TVP-20 should be located near the coaxial cable you wish to use and the RVP-20 near your monitors or the place you wish to bring the video signal to. Select a position that gives you the best access to cable the system and reduces the labor in installation.

HOW TO CABLE THE TVP-20

Connect the video cable you wish to use to the "CAMERA A INPUT" BNC connector and connect the cable going to the RVP-20 to the "CABLE OUTPUT" BNC connector. It is not necessary for power to be on at this time, the video path will only be interrupted during the cable attachment. Next connect the Second camera video output to the "CAMERA B INPUT". BE SURE TO TERMINATE THE END OF THE VIDEO CABLE WITH A 75 OHM TERMINATION OR PROPERLY TERMINATE INTO OTHER EQUIPMENT.

TVP-20.ISB								PAGE 3 OF 4

HOW TO CABLE THE RVP-20

Connect the video cable you are using to the "CABLE INPUT" BNC on the TVP-20 and the "CAMERA A" BNC goes to the monitor, recorder or other video equipment. Connect a cable from the second camera output "CAMERA B" to your monitor, recorder or other video equipment. BE SURE TO TERMINATE THE END OF THE VIDEO CABLE WITH A 75 OHM TERMINATION OR PROPERLY TERMINATE INTO OTHER EQUIPMENT.

POWER SUPPLY INSTALLATION

The TVP-20 and RVP-20 are powered by a 24 VAC wall mount power transformer. Connect the 24 VAC power transformer to the Green terminal block marked AC 24V. At this time you will see the Green LED turn on to indicated power up. A 24VDC power source can also be used if necessary.

OPERATION

The unit provides the ability to transmit two real time video signals over a single coaxial cable. It uses Video-Plexer technology to encode one video signal on top of the other. One transmitter and one receiver is required to complete the system.

CARE AND MAINTENANCE

There is no routine maintenance or calibration required with this equipment. There are no field adjustable controls inside the box. Opening the box will void your warranty.

APPLICATIONS (WHERE TO USE THE SYSTEM)

This system can be used anywhere that a video signal in coax cable exists. Some uses are in a CCTV camera original build, or in an add on camera installation.

APPLICATIONS (LONG DISTANCE VIDEO ON COAX)

The unique method of transmission used by this unit allows you to extend the distance that a video signal can travel on a coax cable. Using the "B" channel of the VIDEO-PLEXER, a video signal from a camera can be transmitted up to 1 mile using ordinary low cost cable line extenders. The key feature of this transmission method is that no degradation of the video frequency response will occur over as much as a mile of transmission distance. Use this method when you have a long way to go to your video monitor location to provide improved video response.

TVP-20.ISB								PAGE 4 OF 4